Batch Manufacturing Record

Section 9.0: In-Process Quality Control (IPQC) Checks for Compression

Product: Aspirin Tablets 325mg

Batch No.: ASP-25-002

Section: 9.0: In-Process Quality Control (IPQC) Checks for Compression

9.1 Personnel Initial Key
All personnel performing checks in this section must record their initials and signature below

Full Name: Anil Sharma

9.2 Sampling & Testing Instructions

• Sampling Frequency: Every 60 minutes

• Sample Size: Take 20 tablets for Weight Variation testing

• From that group, use 5 tablets for Hardness and Thickness testing

• Take an additional quantity of tablets to equal approx. 6.5g for Friability testing

Specification Targets:

|  | Parameter | Specification Target |
| --- | --- | --- |
| 1 | Average Weight | Average Weight: 436.5 mg – 463.5 mg |
| 2 | Hardness | 5.0 – 8.0 kp |
| 3 | Thickness | 3.7 mm – 4.3 mm |
| 4 | Friability | NMT 1.0% |

9.3 IPQC Monitoring Data

| Time of Check | 11:00:00 | 12:00:00 | 13:00:00 | 14:00:00 | 15:00:00 | 16:00:00 |
| --- | --- | --- | --- | --- | --- | --- |
| Appearance | Complies | Complies | Complies | Complies | Complies | Complies |
| --- | --- | --- | --- | --- | --- | --- |
| Individual Wt. 1 (mg) | 450 | 449 | 451 | 448 | 450 | 449 |
| Individual Wt. 2 (mg) | 451 | 450 | 452 | 449 | 451 | 450 |
| Individual Wt. 3 (mg) | 451 | 450 | 452 | 449 | 451 | 450 |
| Individual Wt. 4 (mg) | 452 | 451 | 452 | 451 | 452 | 451 |
| Individual Wt. 5 (mg) | 453 | 452 | 453 | 452 | 453 | 452 |
| Minimum Weight (mg) | 450 | 449 | 451 | 448 | 450 | 449 |
| Maximum Weight (mg) | 453 | 452 | 453 | 452 | 453 | 452 |
| Average Weight (mg) | 451.2 | 450.8 | 452.1 | 449.9 | 451.5 | 450.6 |
| --- | --- | --- | --- | --- | --- | --- |
| Hardness 1 (kp) | 6.8 | 6.5 | 6.9 | 7.1 | 6.7 | 7 |
| Hardness 2 (kp) | 6.8 | 6.5 | 6.9 | 7.1 | 6.7 | 7 |
| Hardness 3 (kp) | 6.8 | 6.5 | 6.9 | 7.1 | 6.7 | 7 |
| Average Hardness (kp) | 6.8 | 6.5 | 6.9 | 7.1 | 6.7 | 7 |
| --- | --- | --- | --- | --- | --- | --- |
| Thickness 1 (mm) | 4.1 | 4 | 4.1 | 4.2 | 4.1 | 4.1 |
| Thickness 2 (mm) | 4.1 | 4 | 4.1 | 4.2 | 4.1 | 4.1 |
| Thickness 3 (mm) | 4.1 | 4 | 4.1 | 4.2 | 4.1 | 4.1 |
| Average Thickness (mm) | 4.1 | 4 | 4.1 | 4.2 | 4.1 | 4.1 |
| --- | --- | --- | --- | --- | --- | --- |
| Friability (%) | 0.0031 | 0.0035 | 0.0033 | 0.003 | 0.0034 | 0.0032 |
| --- | --- | --- | --- | --- | --- | --- |
| Machine Adjustments Made |  |  |  |  |  |  |
| Performed By (Initials) | AS | AS | AS | AS | AS | AS |

Conclusion of IPQC Monitoring

Comments: All in-process checks performed during the compression run are consistently within the predefined specifications. The process remained in a state of control from start to finish.

Reviewed By (Supervisor):

• Name: Sunil Verma

• Signature: Sunil Verma

• Date: 18-Mar-2025