3. Secondary Packaging Run Log

Batch Manufacturing Record

Section 19.0: Secondary Packaging Run Log

Document ID: BMR/ASP325/003      Page: 30 of 40

Purpose: To document the execution of the secondary packaging (cartoning) operation for Batch ASP-25-003, including all operational parameters, in-process checks, and observations.

| Field | Details |
| --- | --- |
| 1. Date of Operation: | 23-Apr-2025 |
| 2. Packaging Area / Room ID: | Packaging Suite P-101 |
| 3. Cartoning Machine ID: | CAR-01 |

19.1 Machine Parameter Verification

Instruction: Verify that the machine is operating at the approved parameters as defined in the setup section.

| Parameter | Approved Setting | Verified By (Operator) |
| --- | --- | --- |
| 1. Machine Speed | 60 cartons/minute | Priya Singh |
| 2. Glue System Temperature | 161 °C | Priya Singh |
| 3. Barcode Scanner | Functional | Priya Singh |

19.2 Main Run Log

| Parameter | Recorded Data |
| --- | --- |
| 1. Start Time of Main Run: | 08:15 |
| 2. End Time of Main Run: | 15:00 |
| 3. Total Cartons Produced (from machine counter): | 8,205 (Note: Final count by reconciliation) |

19.3 Stoppages & Observations Log

| # | Time | Reason for Stoppage / Observation | Corrective Action Taken / Resolved By |
| --- | --- | --- | --- |
| 1 | 11:15 Stoppage | Leaflet feeder magazine empty. | Reloaded leaflet feeder with new bundle of PILs (Lot: PP-PIL-2025-01-790). Verified first 5 cartons after restart. |
| 2 | 13:00 Stoppage | Scheduled operator break. | Line paused as per procedure. |
| 3 | 13:30 Stoppage | Resumed operation after break. | Line restarted. All parameters checked. |

19.4 IPQC Monitoring Log

Sampling Frequency: Every 60 minutes, sample 5 finished cartons.

| # | Time of Check | Correct Leaflet Insertion | Proper Carton Gluing & Flap Closure | Overprint Quality (Batch/Expiry) | Comments / Performed By / Verified By |
| --- | --- | --- | --- | --- | --- |
| 1 | 09:00 | Pass | Pass | Pass | All parameters normal. Priya Singh Amit Sharma |
| 2 | 10:00 | Pass | Pass | Pass | All parameters normal. Priya Singh Amit Sharma |
| 3 | 11:00 | Pass | Pass | Pass | All parameters normal. Priya Singh Amit Sharma |
| 4 | 12:00 | Pass | Pass | Pass | All parameters normal. Priya Singh Amit Sharma |
| 5 | 14:00 | Pass | Pass | Pass | All parameters normal. Priya Singh Amit Sharma |
| 6 | 15:00 | Pass | Pass | Pass | All parameters normal. Priya Singh Amit Sharma |

19.5 Final Sign-off

Conclusion:

The secondary packaging run was completed as per the BMR instructions. A minor, routine stoppage was resolved promptly and documented. All IPQC checks were consistently within specification.

Operator:

• Name: Priya Singh

• Signature: Priya Singh

• Date: 23-Apr-2025

Supervisor Review & Approval:

• Name: Amit Sharma

• Signature: Amit Sharma

• Date: 23-Apr-2025