Batch Manufacturing Record

Section 14.0: Primary Packaging - Line Clearance & Setup

Document ID: BMR/ASP325/003 | Page: 25 of 40

Purpose: Verify area, equipment, and materials are clean, correct, and properly set up before primary packaging of Batch ASP-25-003

|  | Field Name | Data |
| --- | --- | --- |
| 1 | Date of Line Clearance | 22-Apr-2025 |
| 2 | Time of Line Clearance | 08:00 |
| 3 | Packaging Area / Room ID | Packaging Suite P-101 |
| 4 | Blister Packing Machine ID | BPM-01 |
| 5 | Reference to Previous Product | Ibuprofen Tablets 400mg |
| 6 | Reference to Previous Batch No. | IBU-25-017 |

Part A: Area & Equipment Verification

| Checklist Item | Specification / Requirement | Status / Comments | Performed By (Operator) | Verified By (QA) |
| --- | --- | --- | --- | --- |
| 1. Area Clearance | Area must be free of all previous materials, documents, and waste. Verified by Priya Singh, QA: Meena Iyer | Verified | Priya Singh | Meena Iyer |
| 2. Equipment Status | BPM-01 visibly clean. Checked by Priya Singh, QA: Meena Iyer | Checked | Priya Singh | Meena Iyer |
| 3. Equipment Logbook Check | LOG-USE-BPM-01 shows "Clean". Verified by Priya Singh, QA: Meena Iyer | Verified | Priya Singh | Meena Iyer |
| 4. Hoppers & Feed Tracks | Clean and dry. Checked by Priya Singh, QA: Meena Iyer | Checked | Priya Singh | Meena Iyer |
| 5. Environmental Conditions | Temperature, RH, Pressure within limits. Verified by Priya Singh, QA: Meena Iyer | Within Specification | Priya Singh | Meena Iyer |

Part B: Critical Material Verification

| Checklist Item | Specification / Requirement | Status / Comments | Performed By (Operator) | Verified By (QA) |
| --- | --- | --- | --- | --- |
| 1. Bulk Product | Containers must be for Batch ASP-25-003 | Verified | Priya Singh | Meena Iyer |
| 2. Printed Lid Foil | Correct Batch No. (ASP-25-003) and Expiry | Verified, Details Match | Priya Singh | Meena Iyer |
| 3. Base Film | Correct base film (PVC/PVDC-250/60) | Verified | Priya Singh | Meena Iyer |
| 4. Reject Bins | All reject bins empty and clean | Checked | Priya Singh | Meena Iyer |

Part C: Machine Setup Verification

| Parameter | Required Setting | Status / Comments | Performed By (Operator) | Verified By (QA) |
| --- | --- | --- | --- | --- |
| 1. Forming Tool Set ID | FT-ASP-10 | ID Matches BMR | Priya Singh | Meena Iyer |
| 2. Sealing Temperature | 180 °C ± 5 °C | Within Specification (181 °C) | Priya Singh | Meena Iyer |
| 3. Sealing Pressure | 4 Bar ± 0.5 Bar | Within Specification (4.1 Bar) | Priya Singh | Meena Iyer |
| 4. Print Sensor | Aligned and functional | Verified Functional | Priya Singh | Meena Iyer |

Authorization:

Line Cleared By (Production): Amit Sharma | 22-Apr-2025 08:00

Verified By (QA): Meena Iyer | 22-Apr-2025 08:15