Batch Manufacturing Record

Section 7.0: Compression - Machine Trial & Parameter Setup

Instruction: Using a small quantity of the lubricated blend for Batch ASP-25-004, start the machine at a slow speed. Perform In-Process Checks (IPCs) and adjust the machine parameters as necessary until tablets consistently meet the specifications defined below.

7.1 Tablet Quality Specifications (As per Master Specification Document)

|  | Parameter | Specification / Target |
| --- | --- | --- |
| 1 | Appearance: | White, round, biconvex, plain on both sides, free from defects. |
| 2 | Average Weight: | 450 mg ± 3% (436.5 mg – 463.5 mg) |
| 3 | Hardness: | 5.0 – 8.0 kp |
| 4 | Thickness: | 4.0 mm ± 0.3 mm |
| 5 | Machine Speed (Main Run): | 50 RPM |

7.2 Machine Parameter Adjustment Log

Instruction: Record all adjustments made during the trial run.

|  | Time | Parameter Adjusted | New Setting | Observation / IPC Result |
| --- | --- | --- | --- | --- |
| 1 | 09:45:00 | Initial Setting | Run briefly to check initial output | REJECTED |
| 2 | 10:00:00 | Fill Depth | Increased fill depth to raise tablet weight | Avg. Wt: 452.1 mg, Hardness: 4.2 kp |
| 3 | 10:25:00 | Main Compression Force | Increased force to 25 kN (Final Setting) | Avg. Wt: 451.5 mg, Hardness: 6.5 kp, Thickness: 4.1 mm. All parameters are within specification. |

7.3 Final Approved Machine Parameters for Main Run

Instruction: Record the final, locked-in machine settings that will be used for the main compression run.

|  | Machine Parameter | Final Approved Setting |
| --- | --- | --- |
| 1 | Machine Speed (RPM): | 50 RPM |
| 2 | Feeder Speed (RPM): | 30 RPM |
| 3 | Fill Depth (mm): | 12.0 mm |
| 4 | Pre-Compression Force (kN): | 5 kN |
| 5 | Main Compression Force (kN): | 25 kN |
| 6 | Ejection Height (mm): | 2.5 mm |

Authorization to Commence Main Compression Run

Instruction: To be signed by Production and QA after confirming that the trial tablets meet all specifications and the machine parameters are finalized.

1. Production Verification:

"I have reviewed the machine trial log and confirm that the process parameters are now established and consistently produce tablets that meet all in-process specifications."

• Production Supervisor Name: Sunil Verma
• Signature: Sunil Verma
• Date / Time: 18-May-2025 / 10:35:00

2. Quality Assurance Authorization:

"Trial data reviewed. Authorization is given to start the main compression run for batch ASP-25-004 using the final approved parameters."

• QA Officer Name: Meena Iyer
• Signature: Meena Iyer
• Date / Time: 18-May-2025 / 10:40:00

End of Machine Trial Section